# MyAirHat Fabrication

Last updated July 10, 2025

# Equipment Settings – Unless Otherwise Specified

- Serger – Juki MO-634DEN

  - Knife always down

| Left | Right | Upper | Lower | St Finger | St Length | Diff. Feed |
| --- | --- | --- | --- | --- | --- | --- |
| None | 4 | 4 | 4 | Forward | 2.5 | 1 |

- Sewing Machine – Baby Lock BL35B – has automatic buttonhole maker

  - Straight stitch, A/B, Stitch width 5, stitch length 4, thread tension 4,

# Cylinder Fabrication Instructions

## Cutting Cylinder

- Cut blank 40 wide x 17” high

  - 17” high may shorten later. 1.5” used for drawstring pocket at the bottom (loses ½” due to seam allowance, so 1” folded into a tube can be too small).

- Cut window and bottom notches per template

  - Notch bottom center 45 degree angles 1.5” high for drawstring exits.

  - Window opening is centered along the top. Use pattern that matches shield pattern. Top center is 1.5” below the center of the top edge. Use pinking shears to cut from the center of the window to the fold edge. Fold back and iron a crease.

## Sewing Cylinder

| Left | Right | Upper | Lower | St Finger | St Length | Diff. Feed |
| --- | --- | --- | --- | --- | --- | --- |
| None | 4 | 4 | 4 | Forward | 2.5 | 1 |

### Top

- Serge the top edge, from the Right side.

### Bottom Drawstring Notch

- Serge center drawstring 45 degrees notch, straightening to sew the inside corner

### Window

- Sew the shield over the window. Teflon foot. Be sure to sew from the Wrong side.

- Use pinking shears and small cutting mat to trim excess material folded back inside.

### Back Seam

- Serge the back seam. Sew from the Wrong side. Pin at top to ensure the top edge is perfectly aligned and the seam is solid. Use a few wonder clips (accuracy is faster!)

### Bottom

| Left | Right | Upper | Lower | St Finger | St Length | Diff. Feed |
| --- | --- | --- | --- | --- | --- | --- |
| None | 2 | 4 | 7 | Forward | 2.5 | 1 |

- Serge bottom edge to form drawstring pocket. Still from the Right side

  - Tensions RN 2 LL 7

  - Fold the drawstring hem up 1.5” on Right side, turn over and fold downagain 0.75” on the Wrong side. Iron  to get just a small clean-edge sticking out beyond the fold at the bottom. See photos

  - Then sew from the Wrong side.

  - The. Restore to Tensions RN 4 LL

## Sewing Cylinder to Hat

- Reverse the cylinder (turn it inside out). Check that the back seam is tight.

- Sew the cylinder to the hat brim

  - Use the piping presser foot

  - Mark the front/back of the hat (based on its center seam) and start sewing at the back seam where the back seam will be at the front/back.

  - Go slow. Sew the right side of the cylinder to the bottom of the hat brim. See photos. Be careful to keep the wire in the piping presser foot groove and keep the top edge of the cylinder also on top of the wire in the groove. Rotate to keep the center of the hat straight out from the needles. Be careful not to let the extra cylinder portion under the needle.

## Finish

- Fold back bottom hem and stretch to flatten, iron mostly flat.

- Cut drawstring cord length to 39". Insert drawstring with cord lock on one end and thread through bottom hem using bodkin from  the center opening around to the center opening, through the cord lock and knot each end.

- QA check. Apply Fray Check as needed.

- Ship

# Shield

- The window is 7 mil (0.007”) clear plastic that comes in 12” x 12” squares. Use the pattern to cut two pieces.

- Chamfer the bottom corners of the shield, about 1/8” or less, just so they won’t poke holes in the fabric later. Too big chamfer may cause stitching problems!

# Hat

Currently we are using purchased Beekeeping Hats and removing everything (veils etc.) beneath the hat brim.

## Photos