# MyAirHat Fabrication

Last updated July 4, 2025

# Equipment Settings – Unless Otherwise Specified

- Serger – Juki MO-634DEN

| Left | Right | Upper | Lower | St Finger | St Length | Diff. Feed |
| --- | --- | --- | --- | --- | --- | --- |
| None | 4 | 4 | 4 | Forward | 2.5 | 1 |

- Sewing Machine – Baby Lock BL35B – has automatic buttonhole maker

  - Straight stitch, A/B, Stitch width 5, stitch length 4, thread tension 4,

# Cylinder Fabrication Instructions

## Cutting Cylinder

- Cut blank 40 wide x 17” high per template

  - 17” high may shorten later. 1.5” used for drawstring pocket at the bottom (loses ½” due to seam allowance, so 1” folded into a tube can be too small).

- Cut window and bottom notches per template

  - Notch bottom center 1.5” high and 1/2” wide for drawstring exits.

  - Window opening is centered along the top. Use pattern that matches shield pattern. Top center is 1.5” below the center of the top edge.

  - Mark the final hem line corners and the inside cut lines. Cut out the inside after hem allowances, and cut very carefully the diagonals to the hem line corners.

## Sewing Cylinder

### Top

- Serge the top edge, from the Right side.

  - Knife down. Tensions all 4

### Bottom

- Serge 2 ends and the two notches up 2” each. Serge from Right side

  - Knife up. Tensions all 4

- Serge bottom edge to form drawstring pocket. Still from the Right side

  - Knife down. Tensions RN 2 LL 7

  - Fold the drawstring hem up 0.75” on Wrong side, turn over and fold again 0.75” on the Right side. Iron and use glue if needed to get just a small clean-edge sticking out beyond the fold at the bottom.

  - Then sew from the Wrong side.

### Window

- Fold hem over twice to hide the inner edge. Roll over hems and iron enough to crease.

- Glue to corners of shield with small dab of glue.

- Sew the shield over the window. Teflon foot. Be sure to sew from the Wrong side.

### Back Seam

- Serge the back seam. Sew from the Wrong side. Use wonder clips to ensure the top edge is perfectly aligned and the seam is solid. Pin to keep from sewing across hem.

| Left | Right | Upper | Lower | St Finger | St Length | Diff. Feed |
| --- | --- | --- | --- | --- | --- | --- |
| None | 4 | 4 | 4 | Forward | 2.5 | 1 |

## Sewing Cylinder to Hat

- Reverse the cylinder (turn it inside out). Check that the back seam is tight.

- Sew the cylinder to the hat brim

  - Use the piping presser foot

  - Mark the front/back of the hat (based on its center seam) and start sewing at the back seam where the back seam will be at the front/back.

  - Go slow. Sew the right side of the cylinder to the bottom of the hat brim. See photos. Be careful to keep the wire in the piping presser foot groove and keep the top edge of the cylinder also on top of the wire in the groove. Rotate to keep the center of the hat straight out from the needles. Be careful not to let the extra cylinder portion under the needle.

## Finish

- Fold back bottom hem and stretch to flatten, iron mostly flat.

- Cut drawstring cord length to 39". Insert drawstring with cord lock on one end and thread through bottom hem using bodkin from  the center opening around to the center opening, through the cord lock and knot each end.

- Anita QA check. Apply Fray Check as needed.

- Ship

# Shield

- The window is 7 mil (0.007”) clear plastic that comes in 12” x 12” squares. Use the pattern to cut two pieces.

- Chamfer the corners of the shield after sewing it onto the cylinder, about 1/8” or less, just so they won’t poke holes in the fabric later. Too big chamfer may cause stitching problems!

# Hat

Currently we are using purchased Beekeeping Hats and removing everything (veils etc.) beneath the hat brim.

## Photos